SECTION 08 42 29.23 – SLIDING AUTOMATIC ENTRANCES

GENERAL

- <SPECIFIER:  Edit this specification as required for the project>

SUMMARY

Furnish and install sliding automatic aluminum/glass entrance system(s).

REFERENCES

UNDERWRITERS LABORATORIES (UL):

UL 325 – Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems.

American National Standards Institute (ANSI) / Builders Hardware Manufacturers Association (BHMA):

ANSI/BHMA A156.10: American National Standard for Power Operated Pedestrian Doors.

ANSI Z97.1 Standards for Safety Glazing Material Used in Buildings.

American Society for Testing and Materials (ASTM):

ASTM B221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

[ASTM E283 – Standard Test Method for Determining Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen.]

American Association of Automatic Door Manufacturers (AAADM).

National Fire Protection Association (NFPA):

NFPA 101 – Life Safety Code.

NFPA 70 – National Electric Code.

[Canadian Standards Association (CSA).]

CAN/CSA-C22.2 No. 247 – Operators and Systems of Doors, Gates, Draperies, and Louvers.

International Code Council (ICC):

[IBC:  International Building Code.]

[CBC: California Building Code.]

National Association of Architectural Metal Manufacturers (NAAMM):

Metal Finishes Manual for Architectural and Metal Products.

American Architectural Manufacturers Association (AAMA):

AAMA 607.1 - Clear Anodic Finishes for Architectural Aluminum.

DEFINITIONS

SO Panel:  Swing-out sidelite shall be able to swing out (for emergency egress).

SX Panel:  Swing-slide panel shall be able to swing out 90 degrees from any point of slide travel (for emergency egress).

O Panel:  Fixed sidelite that is NOT able to swing out.

X Panel:  Sliding panel that is NOT able to swing out.

P: Unit is surface applied to outside face of the wall.

Configurations:

Single Slide Entrance:  Entrance will have one (1) sliding panel; sliding on either the inside or the outside of the sidelite.

Bi-Parting Entrance:  Entrance will have two (2) sliding panels moving in opposite directions; sliding on either the inside or the outside of the sidelites.

Single Slide Telescopic Entrance:  Entrance will have two (2) sliding panels; sliding on either the inside or the outside of the sidelite.

Bi-Parting Telescopic Entrance:  Entrance will have four (4) sliding panels, two panels sliding in opposite directions; sliding on either the inside or the outside of the sidelites

Full Breakout: Swing-slide panel(s) installed to slide along the inside of the swing-out sidelite(s).  Both the sliding panel(s) and sidelite(s) shall be able to swing out 90 degrees from any point of slide travel (for emergency egress).

Fixed Sidelite:  Swing-slide panel(s) installed to slide along the outside of the fixed sidelite(s).  Only the sliding panel(s) shall be able to swing out 90 degrees from any point of slide travel (for emergency egress).

Clear Door Opening:

Single Slide:  When the sliding panel is in the full open position, the clear opening is measured between the leading stile to the jamb (or jamb stop if present).

Bi-Parting:  When the sliding panels are in the full open position, the clear opening is measured between the leading stiles of the two sliding panels.

PERFORMANCE REQUIREMENTS

Temperature Change:

Minus 30-degree Fahrenheit (minus 34 degree Celsius) to 130-degree Fahrenheit (54 degree Celsius).

SUBMITTALS

Product Data: Provide complete product and installation documentation as provided by the manufacturer.

Shop Drawings: Provide shop drawings including elevations, sections, and details of entrance construction.  Include profiles, dimensioned layout, operator, motion /presence sensor control devices, indicating assembly, finish, glazing, electrical, and anchoring requirements.

Informational Submittal:  Manufacturer's product information indicating the  available sustainability credits that can contribute towards LEED certification.

Manufacturer's certificate indicating percentage of pre-consumer, post-consumer, and new content in the extruded aluminum.

Provide manufacturer’s operating and maintenance manual.  The manual to include the name, address, and contact information of the manufacturer providing the entrance and their nearest service representatives.  A spare parts list shall also be included.

Provide manufacturer’s warranty documentation.

QUALITY ASSURANCE

Manufacturer must have a minimum of five (5) years experience in the fabrication of sliding automatic aluminum-and-glass entrance assemblies similar to those specified.

Sliding automatic entrances shall be obtained solely through an authorized trained and factory certified provider.

Sliding automatic entrances shall be certified to meet performance design criteria of the following standards.

ANSI/BHMA 156.10

NFPA 101

UL 325

cUL

IBC

Installation shall be approved by an AAADM certified inspector.

FIELD CONDITIONS

Shop drawings must be reviewed and confirmed by the General Contractor.  Verify actual dimensions of openings to receive automatic entrances by field measurements before fabrication and indicate on shop drawings.

Rough openings must be verified to be plumb, straight, and secure.

General Contractor to make door installer aware of any non-conforming conditions or equipment not indicated on the shop drawings.

Coordinate power requirements, layout, and connections to power supplies with installation of automatic entrances.

WARRANTY

Project Warranty:  Sliding automatic entrance shall be warranted against defect in material and workmanship for a period of two years from the date of manufacturer shipment.

During the warranty period a factory-trained technician shall perform service and repairs. An inspection shall be performed after each adjustment or repair.

During the warranty period all warranty work, including but not limited to emergency service, shall be performed during normal business hours.

PRODUCT

MANUFACTURER

- Record-USA

- Monroe, North Carolina, USA

- (800) 438-1937

SLIDING AUTOMATIC ENTRANCES

Sliding automatic entrances shall be furnished with all required components for a complete installation.  Entrances shall include the following:

Framing.

Header.

Sliding panel(s).

Sidelite panel(s).

Carrier assemblies.

Noise isolating roller track.

Belt drive operator.

Activation and safety devices.

Threshold and guide tracks.

Basis of Design:  Record 5100 Series Sliding Automatic Entrance with stile and rail panels.

- <SPECIFIER:  Select entrance configuration(s) as required for the project>

[Configuration:  SO-SX (Left), SX-SO (Right);  Single slide, full breakout.]

The swing-slide panel slides along the interior side of the swing-out sidelite (one sliding panel and one sidelite).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panel and sidelite.

Mounting:  Between jambs.

[Configuration:  SO-SX-SX-SO;  Bi-parting, full breakout.]

The swing-slide panels slide along the interior side of the swing-out sidelites (two sliding panels and two sidelites).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panels and sidelites.

Mounting:  Between jambs.

[Configuration:  SO-SX-SX (Left), SX-SX-SO (Right);  Telescopic single slide, full breakout.]

Two swing-slide panels slide along the interior side of the swing-out sidelite (two sliding panels and one sidelite).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panels and sidelite.

Mounting:  Between jambs.

[Configuration:  SO-SX-SX-SX-SX-SO;  Telescopic bi-parting, full breakout.]

Two swing-slide panels (four sliding panels) slide along the interior side of each swing-out sidelite (four sliding panels and two sidelites).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panels and sidelites.

Mounting:  Between jambs.

[Configuration:  O-SX (Left), SX-O (Right);  Single slide, fixed sidelite.]

The swing-slide panel slides along the exterior side of the fixed sidelite (one sliding panel and one sidelite).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panel.

Mounting:  Between jambs.

[Configuration:  O-SX-SX-O;  Bi-parting, fixed sidelite.]

The swing-slide panels slide along the exterior side of the fixed sidelites (two sliding panels and two sidelites).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panels.

Mounting:  Between jambs.

[Configuration:  O-X-SX (Left), SX-X-O (Right);  Telescopic single slide, fixed sidelite.]

Lead swing-slide panel and secondary slide panel slide along the exterior side of the fixed sidelite (two sliding panels and one sidelite).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panels.

Mounting:  Between jambs.

[Configuration:  O-X-SX-SX-X-O;  Telescopic bi-parting, fixed sidelite.]

Lead swing-slide panel and secondary slide panel slide along the exterior side of each fixed sidelite (four sliding panels and two sidelites).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panels.

Mounting:  Between jambs.

[Configuration:  P-SX (Left), SX-P (Right);  Single slide, surface mounted.]

The swing-slide panel slides along the exterior side of the wall (one sliding panel).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panel.

Mounting:  Surface mounted header installed on face of wall.

[Configuration:  P-SX-SX-P;  Bi-parting, surface mounted.]

The swing-slide panels slide along the exterior side of the wall (two sliding panels).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panels.

Mounting:  Surface mounted header installed on face of wall.

[Configuration:  P-X-SX (Left), SX-X-P (Right);  Telescopic single slide, surface mounted.]

Lead swing-slide panel and secondary slide panel slide along the exterior side of wall (two sliding panels).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panels.

Mounting:  Surface mounted header installed on face of wall.

[Configuration:  P-X-SX-SX-X-P;  Telescopic bi-parting, surface mounted.]

Lead swing-slide panel and secondary slide panel slide along the exterior side the wall (four sliding panels).

Traffic Pattern:  [One] [Two] way.

Emergency Breakout:  Sliding panels.

Mounting:  Surface mounted header installed on face of wall.

ENTRANCE COMPONENTS

Aluminum: Alloy and temper recommended by manufacturer for type of use and specified finish.

Header, frames, stiles and rails:  Alloy 6063-T5.

Extruded bars, rods, profiles and tubes:  ASTM B221.

Sheet and plate:  ASTM B209.

Sliding Panels and Sidelites: Shall have 1-3/4” thick extruded aluminum stiles and rails, with a minimum 0.125 inch (3.2 mm) structural wall thickness. Stiles and rails shall be mechanically fastened with mortise and tenon blocks.

- <SPECIFIER:  Sight line width of the vertical stiles is an additional 3/4" (19 mm) due to the width of the glazing stop>

Vertical Stile Width:  [Narrow stile, 2” (51 mm).]  [Medium stile, 3-1/2” (89 mm).]  [Wide stile, 5” (127 mm).]

- <SPECIFIER:  Sight line width of horizontal rail is an additional 1-1/2" (38 mm) due to the width of the glazing stops>

- <SPECIFIER:  3-1/2” horizontal rail required with exit device.  Sight line width of horizontal rail is 5” (127 mm)>

[Horizontal Rail (Muntin Bar) Height:  [1-1/2” (38 mm).]   [2-1/2” (64 mm).]   [3” (76 mm).]  [3-1/2” (89 mm).]  [4” (102 mm).]  [5” (127 mm).] [6” (152 mm).]

- <SPECIFIER:  Sight line width of bottom rail is an additional 1-1/2" (38 mm) due to the width of the glazing stops>

- <SPECIFIER:  Narrow stile doors are available with all bottom rail options.  Medium stile doors are available with 6” and 10” bottom rails.  Wide stile doors are available with 10” bottom rail only>

Bottom Rail Height:  [3-1/2” (89 mm).]  [6” (152 mm).]  [10” (254 mm).]

Overlapping stiles shall have interlocks to prevent separation of panels. Interlocks to be adjusted to maintain security when entrance is in the closed and locked position.

Swing-out Panels:  Panels can swing out 90 degrees at any point in the slide movement to provide egress per NFPA 101. Required force to “breakout” panels shall not exceed 50lbf (222N) applied to the lock stile. Entrances with breakout feature are UL Listed as an exit way.

Glazing Material:  Comply with ANSI Z97.1.

- <SPECIFIER:  Select required glass thickness.  Requirements for glazing specified in Division 8 Section “Glazing”.  Glass provided “by others”.>

Sliding Panels and Sidelite Glazing Prep:  [1/4” (6 mm) glass thickness standard.]  [3/8” (9.5 mm) glass thickness.]  [1/2” (13 mm) glass thickness.]  [9/16” (14 mm) glass thickness.]  [5/8” (16 mm) glass thickness.]  [3/4” (19 mm) glass thickness.]  [1” (25 mm) glass thickness.]

See Division 8 Section “Glazing” for glass requirements.

Sliding Panels and Sidelites:  Glass thickness shall be the same for both the sliding panels and the sidelite panels.

Exterior Glass Stop Extrusion:  Sliding panels and sidelites shall be provided with non-removable security glass.

Framing Members:  Extruded aluminum, minimum 0.125 inch (3.2 mm) thick and reinforced as required to support imposed loads.

[Single Slide and Bi-Parting Entrances:  [1-3/4” x 4-1/2” (44 mm x 114 mm).]  [1” x 4-1/2” (25 mm x 114 mm).]

[Telescopic Single Slide and Bi-Parting Entrances:  [1-3/4” x 6-3/4” (44 mm x 171 mm).]

[Transom shall be as indicated on the drawings.]

Header:  Extruded aluminum header extending the full width of sliding entrance, installed between the jambs.

[Single Slide and Bi-Parting Entrances:  4-1/2” wide x 7-3/4” high (114 mm wide x 197 mm high).]

[Telescopic Single Slide and Bi-Parting Entrances:  6-3/4” wide x 7-3/4” high (171 mm wide x 197 mm high).]

The header shall have removable access panel for servicing of door operator and control.

Roller Track:  Replaceable, continuous extruded aluminum track.

Track covered with rubber compound.

Track captured within an extruded channel in the header.

Neoprene isolation to reduce noise and vibration.

Sliding Panel Carrier Assembly:  Each sliding panel shall be supported by two (2) carrier assemblies that allow vertical adjustment, each carrier having a minimum of two supporting rollers and one adjustable “anti-riser” roller.  The carrier assemblies shall travel along a horizontal plane on the replaceable extruded aluminum.

Overhead Rollers:  Carrier assembly consists of polymer type self-lubricating steel ball bearing rollers.

Minimum of two (2) load bearing 2-inch (51 mm) diameter rollers and one (1) anti-rise roller per carrier assembly, two carrier assemblies per sliding leaf.

HARDWARE

Provide units in sizes and types recommended by sliding automatic entrance manufacturer for uses indicated.

Breakaway Devices:  Pivot assemblies that allow swing-out sidelites and swing-slide panels to swing in the direction of egress to full 90 degrees from any operating position.

Spring-loaded adjustable ball detent shall retain swing-slide panels and swing-out sidelites in a closed position.

- <SPECIFIER:  The door closer is standard on the fixed sidelite entrance and optional on the full breakout entrance>

[Door Closer:  Door closer concealed in the top rail to stop the swing-slide panels and swing-out sidelites from opening beyond 90 degrees and to return the swing-out panels to the closed position.]

Locking Devices:

[Mortise Lock Device:  ANSI A156.5, Grade 1 lock installed in the strike rail.]

- <SPECIFIER:  The Adams Rite 4550 lever is not available on the fixed sidelite entrances>

[Hook Bolt Latch:  Laminated steel hook bolt that latches into the jamb or strike stile (single slide entrances).]

Operated by keyed cylinder on exterior side.

Operated by [thumb turn]  [Adams-Rite 4550 lever] on interior side.

[Lock indicator provided on interior side.]

[Two-Point Locking Device:  Hook bolt latches into the adjacent strike stile or jamb and extends a concealed vertical flush bolt into the overhead carriage assembly (single slide or bi-parting entrances).]

Operated by keyed cylinder on exterior side.

Operated by [thumb turn]  [Adams-Rite 4550 lever] on interior side.

[Lock indicator provided on interior side.]

[Three-Point Locking Device:  Hook bolt latches into the adjacent strike stile and extends a concealed vertical flush bolt into the overhead carriage assembly and into the threshold (bi-parting entrances).]

Operated by keyed cylinder on exterior side.

Operated by [thumb turn]  [Adams-Rite 4550 lever] on interior side.

[Lock indicator provided on interior side.]

Keyed Cylinders:

[Manufacturer’s standard keyed cylinder.]

[Keyed cylinder as specified in Division 8 Section “Door Hardware”.]

[Keyed cylinder compatible with Owner’s existing key  system.]

[Keyed cylinder by others.]

- <SPECIFIER:  Exit device is recessed in a 3-1/2” horizontal rail.  Sight line width of the horizontal rail is 5” (127 mm)>

[Exit Device:  Recessed mounted exit device with a concealed vertical flush bolt latching to the overhead carriage assembly to secure the breakout function.  Exit device installed on leading swing-slide panels, recessed in a 3-1/2 inch (89 mm) horizontal rail.]

[Electrified Lock:  Concealed electric lock to secure the sliding function of the swing-slide panels.  Electric lock is mounted in the entrance header.]

[Fail Secure Lock:  Upon loss of power, the manual sliding function remains locked.]

[Fail Safe Lock:  Upon loss of power, the manual sliding function is unlocked.]

[Alarm Contacts:  Factory installed alarm contacts for remote monitoring.]

- <SPECIFIER:  The master control can provide the same signal as the alarm contacts.  The alarm contacts are a redundant means to monitor the door status if desired>

[Door Open]

[Door Closed]

[Door Breakout Status]

[Electric Lock Status]

Weatherstripping:  Replaceable weatherstripping around the perimeter of all sliding panels and sidelites to reduce energy loss.

- <SPECIFIER:  Select GreenSeal when air leakage is to be 1.00 cu ft/min sq ft when tested at a pressure of at least 1.57 pounds per square foot in accordance with ASTM E283.  Not available for telescopic or surface mount configurations without sidelite panels.>

[Manufacturer’s standard weatherstripping includes the following:

Double pile weatherstripping on the strike rail of sliding panel(s).

Single pile weatherstripping at the following locations:

Between the carriage assembly and header.

Lead stile of sidelite(s).

Pivot stile of sidelite(s).

Adjustable nylon sweep surface applied to sliding panel(s).]

[GreenSeal weatherstripping includes the following:

Double three fin vinyl weatherstripping at the following locations:

Strike rail of sliding panel(s).

Pivot stile of sidelite(s).

Single three fin vinyl weatherstripping at the following locations:

Lead stile of sidelite(s).

Pivot stile of sliding panel(s).

Single pile weatherstripping at the following locations:

Between the carriage assembly and header.

T- Seal at the following locations:

Between the carriage assembly and sliding panel(s).

Surface applied bubble seal at the following locations:

Between header and sidelite(s).

Adjustable nylon sweep surface applied to panels.]

Guide Track/Threshold:  Manufacturer’s sway rail/threshold as indicated.

- <SPECIFIER:  Threshold with pin guides is used with full breakout entrances, SO-SX or SO-SX-SX-SO configurations; and with telescopic full breakout entrances, SO-SX-SX or SO-SX-SX-SX-SX-SO configurations>

[Threshold with Pin Guides:  1/2 inch (12.7mm) high continuous aluminum threshold with integral pin guide track(s) shall span the width of the sliding entrance header and fit between the vertical framing members.  Threshold shall meet ADA requirements and include ramps where required for the floor conditions indicated.]

- <SPECIFIER:  Sway rail with threshold is used with fixed sidelite entrances, O-SX or O-SX-SX-O configurations; with surface mounted entrances, P-SX or P-SX-SX-P configurations; and with fixed sidelite telescopic entrances, O-X-SX or O-X-SX-SX-X-O configurations>

[Sway Rail With Threshold:  [1/2 inch (12.7mm)]  [1/4 inch (6mm)] high continuous aluminum threshold shall span the width of the sliding entrance header and fit between the vertical framing members.  Sway rail is attached to threshold to guide sliding panels but shall not extend into clear door opening.  Threshold shall meet ADA requirements and include ramps where required for the floor conditions indicated.]

- <SPECIFIER:  Sway rail without threshold is used with fixed sidelite entrances, O-SX or O-SX-SX-O configurations; with surface mounted entrances, P-SX or P-SX-SX-P configurations; and with telescopic fixed sidelite entrances, O-X-SX or O-X-SX-SX-X-O configurations>

[Sway Rail Without Threshold:  Sway rail is attached to floor to guide sliding panels but shall not extend into clear door opening.]

DOOR OPERATORS AND CONTROLS

Motor Gearbox Assembly:  Sliding panel movement shall be driven by a sealed, low voltage class II, 1/8 horsepower 24v DC motor and gearbox and nylon reinforced drive belt.  The motor shall have a current draw of not more than 100 watts.  The sealed motor gearbox assembly shall be capable of driving sliding panels up to 220 lbs.  A second motor gearbox can be utilized on the same application giving a capability of moving sliding panels weighing up to 440 lbs.  The motor gearbox assembly shall be mounted directly to the header extrusion.

Master Control:  The master control shall be capable of being programmed by either the S.M.A.R.T. panel or by an application loaded on a smart device which is accessed with a dongle plugged into the operator. Both the S.M.A.R.T. panel and the Application are capable of programming all swinging, sliding and folding doors within the Record product offering. The master control shall have only digitally adjustable parameters (for repeatability purposes, employing potentiometers as a method of setting parameters shall not be allowed). The master control shall be a microprocessor capable of being programed for the control settings and the following:

Opening and closing speeds

Acceleration

Door open time delay

Remote door open time delay

Partial opening size

Reverse adjust sensitivity

Control of emergency battery back-up utilities

Fire alarm signals

Directional traffic flow

Locking

Remote input commands

Detect faults, including type of fault, and responds to them.

Sends data to the S.M.A.R.T. panel indicating that there is a fault.

Provides user with a telephone contact of the authorized service agent.

Updates to the software can be loaded and updated using the Application and dongle.

MOTION AND PRESENCE SENSORS

General:  Provide the types of activation and safety sensors specified, installed and adjusted in accordance with ANSI/BHMA A156.10 standards for the condition of exposure and for long-term, maintenance-free operation under normal traffic load for type of occupancy indicated.  Coordinate activation and safety devices with entrance operation and door operator mechanisms.

Combined Activation and Safety Sensor: Shall be 24 VDC with Class ll circuits.

The combined activation and presence sensor shall use microwave technology to detect motion and active infrared technology to detect presence.  The sensor shall be centrally located in the clear door opening, header-mounted on each side of the entrance. The sensor for activation and the sensor for presence shall be housed in one unit.

Hold-Open Beams:

Infrared photoelectric beams shall be mounted in the vertical stiles of the sidelite or the vertical jambs. The photoelectric beams are mounted at both 24” and 48” above finished floor.  Breaking either emitter beam will cause the sliding panels to open, remain open until the entrance is cleared and then the sliding panels will close.  After closing the entrance will be fully functional.

ELECTRICAL

The sliding automatic entrance shall consume no more than 100W of electricity at full load power.

Power:  120 VAC, 1 phase, 5 amp dedicated circuit per sliding automatic entrance shall be provided to the header by others.

[Battery Back-up:  A header mounted UPS (uninterruptible power supply) that provides 300 opening cycles.]

FABRICATION

General:  Factory fabricate automatic entrance components to designs, sizes, and thicknesses indicated and to comply with indicated standards.

Miscellaneous hardware:  Factory installed per project requirements.

Framing:  Prefabricated to be assembled in field.

Door operator:  Factory installed in header.

Glazing:  Prefabricated glazing stops included per customer specification.

ALUMINUM FINISHES

Comply with NAAMM’s “Metal Finishes Manual for Architectural and Metal Products” for applying and designing finishes. Finish designations beginning with “AA” comply with requirements established by American Architectural Manufacturers Association (AAMA).

- <SPECIFIER:  Select finish as required for project>

[Clear Anodized:  Architectural Class II Anodized Coating (AA-MI2C22A31).]

[Clear Anodized:  Architectural Class I Anodized Coating (AA-MI2C22A41).]

[Dark Bronze Anodized:  Architectural Class I Anodized Coating (AA-MI2C22A44).]

[Medium Bronze Anodized:  Architectural Class I Anodized Coating (AA-MI2C22A44).]

[Light Bronze Anodized:  Architectural Class I Anodized Coating (AA-MI2C22A44).]

[Champagne Anodized:  Architectural Class I Anodized Coating (AA-MI2C22A44).]

[Black Anodized:  Architectural Class I Anodized Coating (AA-MI2C22A44).]

[Kynar Paint Coating:  As selected by Architect from full range of industry colors and color densities.]

[Powder Coat:  To match Architect’s sample.]

[Stainless Steel Clad:  [Polished, non-directional finish.]  [Satin finish.]

[Brass Clad:  [Polished, non-directional finish.]  [Satin finish.]

EXECUTION

INSPECTION

Inspect frame opening for correct size, plumb and square and level floor for safe and reliable performance.

Examine roughing-in for electrical source power to verify actual locations of wiring connections.

Provide written notification to the appropriate personnel of conditions not acceptable to the installer and/or manufacturer. Proceed with installation only after necessary corrections are made to ensure a suitable opening.

INSTALLATION

General:  Install automatic entrances according to manufacturer's written instructions and cited ANSI/BHMA standard, including signage, controls, wiring, and connection to the building's power supply.

Do not install damaged components.  Fit frame joints to produce hairline joints free of burrs and distortion.  Rigidly secure nonmovement joints.  Seal joints watertight.

Install sliding entrance unit plumb, square, and level in properly prepared and supported opening, using specified fasteners, as required by installation instructions, and as detailed on the shop drawings.

Install surface-mounted hardware using concealed fasteners to greatest extent possible.

Set headers, carrier assemblies, tracks, operating brackets, and guides level and true to location with anchorage for permanent support.

Door Operators:  Connect door operators to electrical power distribution system.

Access-Control Devices:  Connect access-control devices to access-control system as specified in Division 28 Sections “Electronic Safety and Security”.

Glazing:  Glaze sliding automatic entrance sliding panels and sidelite panels in accordance with the Glass Association of North America (GANA) Glazing Manual, published recommendations of glass product manufacturer, and published instructions of automatic entrance system manufacturer.

Sealants:  Comply with requirements specified in division 7 Section “Joint Sealants” to provide a weathertight installation.

Set thresholds, bottom guide and track systems and framing members in full bed of sealant.

Seal perimeter of framing members with sealant.

Signage:  Apply signage on both sides of each sliding panel and sidelite as required by the ANSI/BHMA standard and manufacturers installation instructions.

ADJUSTING

Adjust hardware, moving parts, door operators, and controls to function smoothly, and lubricate as recommended by manufacturer; comply with requirements of applicable ANSI/BHMA standards.

Verify installation and alignment of all entrance weatherstripping as required for weathertight closure and specified air infiltration requirements.

FIELD QUALITY CONTROL

An AAADM certified technician shall inspect and approve automatic entrances for compliance with the ANSI/BHMA standards before placing the entrances into operation. The certified technician shall be approved by the manufacturer.

CLEANING

Clean glass and metal surfaces promptly after installation.  Remove excess glazing and sealant compounds, dirt, and other substances.  Repair damaged finish to match original finish.

Clean adjacent surfaces soiled by door installation.

DEMONSTRATION

Engage a factory-authorized service representative to train Owner's maintenance personnel to adjust, operate, and maintain automatic entrances.

- END OF SECTION 08 42 29.23